SIMATS ENGINEERING

ASSESSMENT TOOL 1

COURSE NAME: COMPUTER ARCHITECTURE COURSE CODE: CSA1202

NAME :YAZHINI.E.M

REG NO:192511165

Computer Organization & Architecture

Assignment: CPU, Memory, I/O Interaction, Bus Structures, Instruction Cycle, CPI & 8085 Addressing Modes

# Question 1: Smart City Surveillance System — Bus Structures & CPU–Memory–I/O Interaction

## 1.1 Restating the Problem

A smart-city surveillance network has dozens of cameras continuously streaming video. Each frame must travel from the camera (I/O device) into main memory, and the CPU must periodically read frames from memory, run detection algorithms, and write results/alerts back. During peak hours (e.g., rush hour, a public event) the volume of simultaneous transfers rises sharply, and the system starts lagging — threats are detected late.

## 1.2 How CPU, Memory and I/O Interact

Every one of the three subsystems is connected through a system bus consisting of three logical parts:

- Address Bus — carries the memory/I/O address the CPU wants to access (unidirectional, CPU → others).

- Data Bus — carries the actual data being transferred (bidirectional).

- Control Bus — carries control/timing signals such as Read, Write, Interrupt Request, DMA Request, Ready/Wait (bidirectional).

For a surveillance workload, three kinds of transfers are constantly competing for these buses:

- I/O → Memory: incoming video frames from each camera controller are written into buffers in RAM (often via DMA).

- Memory → CPU: the CPU fetches frame data and its own instructions from memory to run detection/analysis.

- CPU → I/O: control signals / alert data are sent out to actuators, alarms, or storage devices.

Because CPU speed (GHz) is far higher than I/O device speed and memory access is also slower than the CPU, a queue builds up whenever many of these transfers must share one physical pathway. This mismatch — combined with contention for a shared bus — is the direct technical cause of the lag observed in the surveillance system.

## 1.3 Single-Bus Architecture — Why It Struggles Under Load

In a single-bus (shared bus) architecture, the CPU, memory and all I/O devices are connected to one common set of address/data/control lines.

Figure 1.1 — Single shared bus: CPU, Memory and I/O all contend for one path

- Simplicity & low cost: fewer wires, simpler controller design, easy to expand with more devices.

- Bottleneck: only ONE transfer can occur on the bus at any given clock cycle. If cameras are flooding the bus with frame-write requests, the CPU's own fetch of the next instruction/data must WAIT.

- Bus arbitration overhead: an arbiter must grant bus ownership to one requester at a time (fixed priority / round robin / daisy chaining), which itself adds delay under heavy contention.

Under peak surveillance load this produces exactly the symptom described: growing wait-states, delayed frame delivery to the CPU, and hence late threat detection.

## 1.4 Multi-Bus Architecture — The Improvement

A multi-bus (multiple-bus) architecture provides separate, often simultaneous, pathways — typically a dedicated high-speed memory bus between CPU and RAM, and a separate I/O bus (sometimes bridged through an I/O controller/cache), possibly with a DMA controller managing I/O transfers independently of the CPU.

Figure 1.2 — Multi-bus architecture: dedicated memory bus and I/O bus operate in parallel

- Parallelism: the CPU can fetch instructions/data on the memory bus WHILE camera frames are simultaneously being DMA-transferred into memory on the I/O bus.

- Reduced contention: since traffic is distributed across multiple buses, peak-hour camera traffic no longer directly stalls every CPU memory access.

- Scalability: more cameras/devices can be added on the I/O bus without proportionally slowing down CPU–memory traffic.

- Trade-off: higher hardware cost and more complex synchronization/arbitration logic between buses.

## 1.5 Comparison

| Aspect | Single-Bus | Multi-Bus |
| --- | --- | --- |
| Transfers at once | Only one | Multiple, in parallel |
| Cost & complexity | Low | Higher (more controllers/wiring) |
| Performance under heavy I/O load | Degrades sharply (contention) | Sustained (I/O and CPU paths separated) |
| Best suited for | Small / low-traffic embedded systems | High-throughput real-time systems (e.g. surveillance) |

## 1.6 Role of Improving the Instruction Execution Cycle

Beyond the bus, the CPU's own fetch–decode–execute cycle can be optimized so each frame-analysis instruction completes faster and frees the bus/CPU sooner:

- Pipelining: overlap fetch of the next instruction with decode/execute of the current one, increasing instruction throughput.

- Cache memory: keep frequently used detection-model data/instructions close to the CPU, cutting the number of slow main-memory accesses (hence less bus traffic).

- DMA (Direct Memory Access): let camera controllers write frames directly into memory without CPU intervention on every byte, freeing the CPU/bus for analysis work.

- Interrupt-driven I/O instead of polling: CPU is notified only when a frame is ready, instead of wasting cycles repeatedly checking device status.

- Wider data bus / burst transfers: move larger chunks of frame data per bus cycle, reducing the number of arbitration rounds needed.

## 1.7 Conclusion (Q1)

The lag during peak hours is caused by bus contention among concurrent CPU, memory and I/O transfers, worsened by a single shared bus. Moving to a multi-bus architecture with dedicated memory/I/O paths, combined with DMA, caching, interrupt-driven I/O and pipelined instruction execution, removes the bottleneck and restores real-time responsiveness for threat detection.

# Question 2: Real-Time Stock Trading System — CPI, Execution Time & Architectural Enhancements

## 2.1 Restating the Problem

A trading processor executes millions of instructions per second, yet users see delays during high load. The diagnosis given is a HIGH CPI (Cycles Per Instruction) driven by frequent memory accesses and branch instructions — both of which are common in transaction-matching / order-book logic (lots of conditional checks and data lookups).

## 2.2 The Performance Equation

CPU execution time is governed by the classic performance equation:

Execution Time = Instruction Count (IC)  x  CPI  x  Clock Cycle Time
 
                 = IC  x  CPI  /  Clock Frequency

Increasing CPI directly and linearly increases execution time for the same instruction count and clock speed — this is exactly why a high CPI causes transaction delays even on a fast, high-frequency processor.

## 2.3 How the Fetch–Decode–Execute Cycle Impacts Execution Time

Figure 2.1 — The instruction cycle repeated for every instruction

- Fetch stage stalls: if the required instruction/data is not in cache, the CPU must wait many extra cycles for a main-memory access (a 'memory stall') — this alone can push CPI from ~1 up to 4–5 in memory-heavy code.

- Decode stage: complex instruction formats (variable-length, multiple addressing modes) take longer to decode, adding cycles.

- Execute stage — branch instructions: every conditional branch (e.g., 'if price > threshold') forces the CPU to determine the next PC only after evaluating a condition. If instructions were already fetched speculatively down the wrong path (pipeline), those fetched instructions must be flushed — a branch misprediction penalty of several cycles.

- Write-back / store: writing results back to memory (e.g., updating an order book entry) can again stall if memory is busy or not cached.

Because millions of instructions are executed per second, even a small average increase in CPI per instruction (extra memory-stall or branch-penalty cycles) multiplies into a large aggregate delay — this is the direct cause of the reported transaction-processing lag.

## 2.4 Methods to Reduce CPI / Improve Performance

- Instruction pipelining — overlaps fetch/decode/execute/write-back of successive instructions so, in steady state, close to one instruction completes per clock cycle instead of four.

- Multi-level caching (L1/L2/L3) — keeps hot order-book data and frequently executed matching-engine code close to the CPU, sharply cutting memory-access latency and hence memory-stall cycles.

- Branch prediction & branch target buffers — predict the likely outcome of conditional branches (e.g., using history of past trade decisions) so the pipeline is rarely flushed.

- Speculative / out-of-order execution — allows independent instructions to execute while a branch or memory operation is still resolving, hiding latency.

- Superscalar / multiple execution units — issue and execute more than one instruction per clock cycle.

- Register renaming & larger register files — reduce false data dependencies that otherwise stall the pipeline.

- Reducing memory access frequency algorithmically — batching/aligning data structures (e.g., order book) to reduce cache misses.

## 2.5 Code Output — Quantifying the Improvement

The Python model below applies the performance equation to a representative batch of 50 million instructions, comparing a naive design (CPI = 4.2, dominated by memory stalls and branch penalties) against the same processor after pipelining, caching, and branch prediction are applied (CPI reduced to 1.15).

def execution_time(ic, cpi, clock_freq_ghz):
    clock_cycle_time_ns = 1 / clock_freq_ghz
    total_cycles = ic * cpi
    return (total_cycles * clock_cycle_time_ns) / 1e6   # -> ms
 
instruction_count = 50_000_000
cpi_before, clock_before = 4.20, 2.0   # naive design
cpi_after,  clock_after  = 1.15, 2.0   # pipelined + cache + branch prediction
 
t_before = execution_time(instruction_count, cpi_before, clock_before)
t_after  = execution_time(instruction_count, cpi_after,  clock_after)
speedup  = t_before / t_after

Program Output:

============================================================
STOCK TRADING SYSTEM - CPU PERFORMANCE ANALYSIS
============================================================
Instruction count per batch : 50,000,000
 
BEFORE  -> CPI = 4.20, Clock = 2.0 GHz
          Execution time    = 105.000 ms
 
AFTER   -> CPI = 1.15, Clock = 2.0 GHz
          Execution time    = 28.750 ms
 
Speedup achieved            = 3.65x faster
Latency reduced by          = 72.6%
============================================================

This confirms that architectural enhancements alone (no change in clock frequency) can cut transaction-processing latency by roughly 73% purely by reducing CPI — validating pipelining, caching and branch prediction as the correct remedy.

## 2.6 Pipelining Effect — Visualized

Figure 2.2 — Non-pipelined (CPI ≈ 4) vs pipelined (CPI → 1) execution of 3 instructions

## 2.7 Conclusion (Q2)

High CPI, caused by frequent memory stalls and branch instructions in the fetch–decode–execute cycle, is the root cause of trading-system delay under load. Pipelining, hierarchical caching, branch prediction, superscalar execution and register renaming together reduce effective CPI, directly lowering execution time per the performance equation and restoring real-time responsiveness.

# Question 3: 8085-Based Temperature Control Unit — Addressing Modes & Reliability

## 3.1 Restating the Problem

An industrial temperature-control unit uses an 8085 microprocessor: sensors provide analog/digital temperature input (via an ADC/I/O port) and actuators (heater/cooler relay) respond based on processed values. Occasionally, incorrect temperature readings are processed — traced to improper use of addressing modes in the assembly program.

## 3.2 Addressing Modes of the 8085 and How They Affect Data Access

Figure 3.1 — 8085 addressing modes and example instructions

| Mode | How Operand Is Located | Example | Typical Use in This System |
| --- | --- | --- | --- |
| Immediate | Data is part of the instruction itself | MVI A, 32H | Loading a fixed threshold/limit value |
| Register | Operand is inside a CPU register | MOV A, B | Moving a sensor value between registers for processing |
| Direct | Instruction contains the full 16-bit memory address | LDA 2050H | Reading/writing a fixed memory location, e.g. last sensor reading |
| Register Indirect | Address of data is held in register pair (usually H-L) | MOV A, M | Reading a value from a lookup/calibration table pointed to by HL |
| Implied / Implicit | Operand is implicit in the opcode (e.g., accumulator) | CMA, RAL, RRC | Bit manipulation of status/actuator control byte |

Each mode determines exactly WHERE the CPU looks for data. Immediate and Register modes are fast and safe (no external address involved). Direct and Register-Indirect modes, however, depend on an address being correct — a small programming mistake here silently redirects the CPU to the WRONG memory cell, without any error being raised by the hardware.

## 3.3 Root Causes of the Faulty Readings

- Uninitialized / stale register pair in Register-Indirect mode: if HL is not (re)loaded with LXI H, addr before a MOV A, M or MOV M, A instruction, the CPU reads/writes whatever address HL happened to hold from a previous operation — producing a garbage 'temperature' value.

- Incorrect direct address in LDA/STA: a typo in the 16-bit address (e.g., LDA 2050H used instead of LDA 2060H) silently accesses the wrong memory cell — no crash occurs, just a wrong value being treated as valid sensor data.

- Confusing Immediate with Direct/Register mode: e.g., mistakenly using MVI A, 50H (loads the literal constant 50H) when the intent was to load the sensor value stored at address 50H, so a fixed dummy value is processed instead of the live reading.

- Not updating the pointer inside a loop: when scanning multiple sensor channels stored in consecutive memory locations, forgetting INX H after each MOV A, M causes the same location to be re-read repeatedly instead of advancing through the table.

- Sign/format mismatch: treating a signed BCD/two's-complement temperature byte as a plain unsigned direct value (or vice-versa) during comparison instructions (CPI, CMP) causes wrong threshold decisions even though the address itself was correct.

## 3.4 Using the 8085 Instruction Set Reliably — Recommended Practice

A corrected fragment reading a temperature sensor via a calibration/lookup table, comparing it to a threshold, and driving an actuator:

; ---- Reliable temperature-check routine (8085) ----
        MVI  A, 00H      ; (Implied/Immediate) clear accumulator
        LXI  H, 3000H    ; (Immediate, loads pointer) HL -> sensor buffer
        MOV  A, M        ; (Register-Indirect) read live sensor byte via HL
        MVI  B, 4BH      ; (Immediate) B = threshold constant (75 decimal)
        CMP  B           ; (Register) compare A with threshold in B
        JC   HEATER_ON   ; branch if reading below threshold
        STA  2050H       ; (Direct) log latest reading to fixed status cell
        JMP  DONE
HEATER_ON:
        STA  2050H       ; (Direct) still log the reading
        MVI  A, 01H      ; (Immediate) actuator-ON command code
        OUT  01H         ; send command to actuator port
DONE:   HLT

- Always initialize HL explicitly with LXI H before any Register-Indirect access — never assume its previous value.

- Double-check every hard-coded 16-bit address used with LDA/STA/SHLD/LHLD against the system's memory map.

- Use Immediate mode ONLY for genuine constants (thresholds, masks, command codes) — never for values that are supposed to come from a sensor/memory location.

- When scanning tables of multiple sensors, increment the pointer (INX H) each iteration and bound the loop with a counter (register B/C + DCR/JNZ).

- Add a simple range/sanity check on the value read (e.g., compare against known min/max valid temperature) before it is allowed to drive an actuator — this catches an addressing-mode error before it causes an unsafe physical action.

## 3.5 Conclusion (Q3)

The 8085's addressing modes give flexible but literal access to data: the CPU will faithfully read whatever address a Direct or Register-Indirect instruction points to, correct or not. Occasional bad temperature readings in this system are best explained by an unrefreshed HL pointer, a wrong hard-coded address, or confusing Immediate with Direct/Indirect access. Disciplined use of the instruction set — explicit pointer initialization, verified addresses, correct mode selection for constants vs. live data, and a sanity check before actuation — ensures accurate and reliable operation of the temperature-control unit.